Architecture Design Document

Humanoid Robot Industrial Inspection POC

Version 1.0  ·  June 2026  ·  C:\huminoid

# 1. System Overview

The Humanoid Robot Inspection POC is a closed-loop simulation system in which a Unitree G1 humanoid robot performs autonomous visual quality inspection at a work table. The system spans four architectural layers: simulation core, vision pipeline, control layer, and data/training pipeline. An optional cloud augmentation path connects to NVIDIA's Cosmos 3 NIM API to enrich synthetic training data with photorealistic industrial textures.

All compute runs locally on a Windows PC with an AMD RX 580 GPU (CPU physics + OpenGL rendering). No NVIDIA GPU is required at runtime — Cosmos 3 is accessed over HTTP only during the offline training phase.

# 2. Architecture Diagram

Figure 1 — Four-layer system architecture showing data flow between components

# 3. Architectural Layers

## 3.1  Simulation Core

MuJoCo 3.9 provides deterministic rigid-body physics at 250 Hz. The simulation scene is defined in two XML files: a standalone version (placeholder capsule robot) and a full version that includes the Unitree G1 MJCF. Both share the same world geometry: a work table, a reject bin, three part bodies with free joints, and a head-mounted camera.

| Component | File | Role |
| --- | --- | --- |
| MuJoCo Engine | mujoco 3.9.0 (pip) | Physics stepping, collision, rendering |
| Standalone scene | scene/inspection_scene_standalone.xml | Self-contained scene for testing without G1 |
| Full scene | scene/inspection_scene.xml | Includes G1 MJCF via <include> |
| Offscreen render | mujoco.Renderer (640×480) | Generates head-cam frames for vision |
| Interactive view | mujoco.viewer.launch_passive | Real-time 3D display for demo |

## 3.2  Vision Pipeline

YOLOv8n (Nano) runs on CPU at ~80 ms/frame. The Detector class wraps ultralytics, loads the best.onnx exported model, and returns DetectionResult objects containing class ID, confidence, and bounding box. Inference is throttled to every 5th simulation step (~10 fps) to avoid saturating the CPU.

| Component | File | Notes |
| --- | --- | --- |
| Detector | vision/detector.py | Wraps YOLO, returns DetectionResult |
| Model (train) | runs/train/weights/best.pt | PyTorch checkpoint |
| Model (runtime) | runs/train/weights/best.onnx | ONNX export — faster CPU inference |
| Annotator | vision/detector.py _draw() | Draws bboxes + HUD on BGR frame |
| OpenCV window | demo.py | Displays annotated head-cam feed |

## 3.3  Control Layer

The inspection controller implements a 5-state finite state machine decoupled from the physics loop. Each call to step(sim_time, frame) advances the state machine and writes target joint angles to data.ctrl[] via the IK solver.

| State | Trigger | Duration | Arm Action |
| --- | --- | --- | --- |
| WATCHING | Entry / return | Continuous | Home pose; YOLO runs every 200 ms |
| APPROACHING | Defect detected | 1.5 s | Blend to reach_table keyframe |
| PICKING | APPROACHING timeout | 1.0 s | Hold reach pose; remove part |
| DROPPING | PICKING timeout | 1.0 s | Blend to drop_bin keyframe |
| RETURNING | DROPPING timeout | 1.2 s | Blend back to home keyframe |

The IK solver (robot/ik_solver.py) operates in one of two modes: Damped Least Squares (DLS) online IK when the G1 MJCF is loaded (7-DOF right arm, λ=0.05, 50 iterations), or keyframe interpolation fallback for the standalone scene. All three keyframe poses (home, reach_table, drop_bin) are smooth-step interpolated over the state duration.

## 3.4  Data / Training Pipeline

Training data is generated entirely from the MuJoCo offscreen renderer, eliminating the need for real-world images. Each frame is automatically labeled by projecting the known part position through the camera model.

| Stage | Script | Output |
| --- | --- | --- |
| Offscreen render | vision/generate_data.py | 2,000 JPEG frames + YOLO .txt labels |
| Cosmos augment | vision/augment_cosmos.py | 500 photorealistic variants (optional) |
| YOLO fine-tune | vision/train_yolo.py | best.pt + best.onnx (50 epochs, CPU) |
| Dataset config | data/dataset.yaml | 3 classes: good / defect_crack / defect_discolor |

## 3.5  Cloud Augmentation (Cosmos 3 NIM)

NVIDIA's Cosmos 3 Diffusion model is accessed via the NIM HTTP API at ai.api.nvidia.com. Synthetic MuJoCo frames are encoded as base64 and sent with an industrial-context prompt. The returned image has photorealistic factory floor textures applied (image-to-image diffusion, strength=0.6). Bounding box labels are preserved unchanged since geometry is not altered. This step is optional but improves sim-to-real generalization.

POST ai.api.nvidia.com/v1/genai/nvidia/cosmos-1.0-diffusion

{"image": "<base64>", "prompt": "industrial factory floor...", "strength": 0.6}

# 4. Runtime Data Flow

Each simulation step (4 ms) executes the following sequence:

2.  MuJoCo physics step (mj_step) — advances simulation by 4 ms

3.  Every 25th step: Renderer.update_scene() + render() → BGR frame (10 fps)

4.  InspectionController.step(sim_time, frame) called each step

5.  If WATCHING and 200 ms elapsed: Detector.detect(frame) → DetectionResult

6.  If defect found: state → APPROACHING; IKSolver.interpolate_to_pose() called

7.  IKSolver.apply_pose() writes to data.ctrl[] → MuJoCo actuator targets

8.  MuJoCo viewer synced via viewer.sync() — renders updated joint positions

9.  OpenCV window updated with annotated frame + state HUD overlay

# 5. Module Interfaces

| Module | Inputs | Outputs | Key Class/Function |
| --- | --- | --- | --- |
| vision/detector.py | BGR frame (np.ndarray) | DetectionResult | Detector.detect() |
| robot/ik_solver.py | target_pos (np.ndarray) | IKResult / pose dict | IKSolver.solve() |
| robot/inspection_controller.py | sim_time, BGR frame | ControllerStatus | InspectionController.step() |
| vision/generate_data.py | MuJoCo model + renderer | JPEG frames + .txt | render_dataset() |
| vision/train_yolo.py | dataset.yaml | best.pt + best.onnx | train() |
| vision/augment_cosmos.py | JPEG frames + API key | Augmented JPEGs | cosmos_augment() |
| demo.py | scene XML path | Viewer + OpenCV window | run_demo() |

# 6. Dependency Graph

demo.py

├── vision/detector.py          (ultralytics, opencv-python)

├── robot/inspection_controller.py

│     ├── robot/ik_solver.py    (mujoco, numpy, scipy)

│     └── vision/detector.py

└── mujoco                      (physics + viewer + renderer)

vision/train_yolo.py

└── ultralytics                 (torch, torchvision)

vision/generate_data.py

└── mujoco, opencv-python, numpy

vision/augment_cosmos.py

└── requests, Pillow            (NVIDIA NIM API — network only)

# 7. File Map

| Path | Type | Description |
| --- | --- | --- |
| scene/inspection_scene_standalone.xml | MuJoCo XML | Self-contained scene — placeholder robot, no G1 MJCF needed |
| scene/inspection_scene.xml | MuJoCo XML | Full scene — includes G1 MJCF via <include> |
| vision/generate_data.py | Python | Offscreen renderer loop; auto-projects YOLO labels |
| vision/augment_cosmos.py | Python | Calls Cosmos NIM HTTP API; polite rate limiting |
| vision/train_yolo.py | Python | YOLOv8n fine-tune; merges augmented images before train |
| vision/detector.py | Python | Detector class; prefers ONNX over .pt at runtime |
| robot/ik_solver.py | Python | DLS IK (G1) + keyframe fallback (standalone) |
| robot/inspection_controller.py | Python | 5-state FSM; spawns/removes parts; drives arm |
| demo.py | Python | Main entry; viewer + OpenCV cam window + spawn loop |
| verify_setup.py | Python | Pre-flight check: imports, scene load, model presence |
| data/dataset.yaml | YAML | YOLO dataset config — 3 classes, train/val split |

# 8. Key Design Decisions

| Decision | Chosen Approach | Alternative Considered | Rationale |
| --- | --- | --- | --- |
| Simulator | MuJoCo 3.x | Isaac Sim | Isaac Sim requires NVIDIA GPU — not available on AMD RX 580 |
| Vision model | YOLOv8n (CPU) | YOLOv8s / MobileNet | Nano: ~80 ms/frame on i7; small enough for CPU real-time |
| IK method | DLS + keyframe fallback | ROS2 MoveIt | No ROS2 dependency; keyframes sufficient for POC pick-place |
| Training data | MuJoCo offscreen render | Real imagery | No real robot needed; 2,000 frames in ~5 min |
| Augmentation | Cosmos 3 NIM (cloud) | Local diffusion model | AMD GPU can't run Cosmos locally; cloud API costs ~$0.02/image |
| Project path | C:\huminoid | E:\huminoid | E:\ drive is full (0 bytes); short path avoids MuJoCo Windows bug |