Humanoid Robot Industrial Inspection

2-Day Proof of Concept — Unitree G1 + MuJoCo

Project path: C:\huminoid     |     Date: June 2026

# 1. Executive Summary

This POC demonstrates a Unitree G1 humanoid robot performing automated visual quality inspection at a simulated industrial work station. The robot's head-mounted camera continuously inspects parts placed on a work table, classifying each as good or defective. When a defect is detected, the robot's right arm reaches to the table, picks up the defective part, and deposits it in a reject bin — closing a full pick-and-place inspection loop entirely in simulation.

The simulation runs in MuJoCo 3.x (CPU physics, AMD-compatible OpenGL rendering) on a Windows PC with an AMD RX 580 GPU. Training data is generated synthetically from the simulator, then optionally enriched with photorealistic industrial textures via NVIDIA's Cosmos 3 cloud API (NIM). The complete demo targets a 2-day build timeline and is designed to convey a clear industrial ROI story.

# 2. Hardware Overview

| Component | Spec | Assessment |
| --- | --- | --- |
| CPU | Intel i7-6700, 4-core 3.4 GHz | Fine for MuJoCo CPU physics |
| RAM | 16 GB DDR4 | Adequate — reduce batch to 4 if pressure |
| GPU | AMD RX 580 8 GB | OpenGL rendering only (no CUDA/NVIDIA) |
| Storage | 466 GB SSD | Use SSD path — HDD too slow for I/O |
| OS | Windows 10 Pro 64-bit | Supported; use short paths for MuJoCo |

Note: Isaac Sim requires an NVIDIA GPU and is not viable on this hardware. MuJoCo is the correct simulator for this setup.

# 3. Tech Stack

| Component | Tool | Rationale |
| --- | --- | --- |
| Simulation | MuJoCo 3.x | CPU physics, OpenGL, AMD-compatible, Windows support |
| Robot model | Unitree G1 MJCF | Official MJCF from unitree_mujoco repo |
| Vision | YOLOv8n (ultralytics, CPU) | Nano model: ~80 ms/frame on i7-6700 |
| Control | Python MuJoCo bindings + IK | No ROS2 needed; DLS IK + keyframe fallback |
| Visualization | MuJoCo viewer + OpenCV | Native 3D viewer + annotated camera window |
| Data generation | MuJoCo offscreen renderer | 2,000 labeled frames with auto-projected YOLO labels |
| Scene augmentation | Cosmos 3 via NVIDIA NIM API | HTTP call to build.nvidia.com — no local GPU needed |

Cosmos 3 (NVIDIA NIM API): Cosmos 3 cannot run locally on AMD hardware, but NVIDIA hosts it as a cloud API. Synthetic MuJoCo frames are sent over HTTP and returned with photorealistic industrial textures/lighting applied. Free trial credits are available at build.nvidia.com; estimated cost ~$0.01–0.05 per image.

# 4. POC Concept: Defect Detection at Work Table

The G1 humanoid robot stands beside a work table. Inspection parts appear on the table one at a time (spawned by the simulation script every 5 seconds). The robot's head camera runs YOLOv8n inference every 200 ms and classifies each part:

| Class ID | Label | Visual Appearance | Robot Action |
| --- | --- | --- | --- |
| 0 | good | Green box | Arm stays at home pose |
| 1 | defect_crack | Red box (crack texture) | Arm reaches, picks, drops in reject bin |
| 2 | defect_discolor | Dark grey box | Arm reaches, picks, drops in reject bin |

Why a table instead of a conveyor: A static work table is simpler to model in MuJoCo in one day. A conveyor requires custom geom physics. The table setup demonstrates identical industrial logic — a cell inspection station — with less implementation risk.

# 5. Repository Structure

All files located at C:\huminoid  (E:\ drive is full — 0 bytes free)

C:\huminoid\

├── scene\

│   ├── inspection_scene_standalone.xml   ← self-contained scene (no G1 needed)

│   └── inspection_scene.xml              ← full scene (requires unitree_mujoco)

├── data\

│   ├── dataset.yaml                      ← YOLO dataset config

│   ├── images\{train,val}\               ← rendered JPEG frames

│   ├── labels\{train,val}\               ← YOLO .txt label files

│   └── augmented\                        ← Cosmos-augmented frames

├── vision\

│   ├── generate_data.py                  ← offscreen render + auto-label

│   ├── augment_cosmos.py                 ← Cosmos NIM API augmentation

│   ├── train_yolo.py                     ← YOLOv8n fine-tune (CPU mode)

│   └── detector.py                       ← live detection class

├── robot\

│   ├── ik_solver.py                      ← DLS IK + keyframe fallback

│   └── inspection_controller.py          ← 5-state inspection machine

├── demo.py                               ← main entry: viewer + control loop

├── verify_setup.py                       ← environment sanity check

└── requirements.txt

# 6. Day 1 Plan

## Step 1 — Python Environment Setup  (1 h)

Install Python 3.11 (avoid 3.12 for MuJoCo compatibility)

Create and activate virtual environment:

python -m venv .venv

.venv\Scripts\activate

pip install -r requirements.txt

Key packages: mujoco>=3.1.0, ultralytics>=8.2.0, opencv-python, numpy, scipy, dm_control, requests

## Step 2 — Get Unitree G1 MJCF Model  (30 min)

Clone the official Unitree MuJoCo repository:

git clone https://github.com/unitreerobotics/unitree_mujoco.git C:\unitree_mujoco

G1 model files located at: C:\unitree_mujoco\unitree_robots\g1\

Fallback: use inspection_scene_standalone.xml (no G1 required) if MJCF has issues

## Step 3 — Build MuJoCo Scene  (2–3 h)

Two scene files are provided — standalone (placeholder robot) and full (G1 MJCF)

Scene includes: work table, reject bin, 3 part bodies, head camera site, 3 lights

Verify standalone scene loads:

python -m mujoco.viewer --mjcf scene\inspection_scene_standalone.xml

## Step 4 — Synthetic Data Generation  (2 h)

Run generate_data.py to render 2,000 labeled frames via MuJoCo offscreen renderer

Randomises: part position on table, light intensity (0.5–2.0×), part colour jitter

Auto-generates YOLO bounding boxes from camera projection matrix

Target distribution: ~600 good / ~700 defect_crack / ~700 defect_discolor

python vision\generate_data.py --n 2000

## Step 4b — Cosmos 3 Augmentation via NIM API  (1 h, optional)

Sign up at https://build.nvidia.com to get an API key (free trial credits included)

Augments 500 of the 2,000 synthetic frames with photorealistic industrial textures

Bridges the sim-to-real gap without a local NVIDIA GPU

$env:NVIDIA_API_KEY = "nvapi-xxxx"

python vision\augment_cosmos.py --n 500 --strength 0.6

## Step 5 — YOLOv8n Training  (1–2 h)

Fine-tunes YOLOv8n on the synthetic dataset; runs entirely on CPU

Expected training time: ~1.5 h for 50 epochs on i7-6700

Exports best checkpoint to ONNX for faster CPU inference

python vision\train_yolo.py --epochs 50 --batch 8 --imgsz 480

# Reduce --batch to 4 if RAM pressure occurs

# 7. Day 2 Plan

## Step 6 — IK Solver for G1 Right Arm  (2–3 h)

robot/ik_solver.py implements two modes:

DLS (Damped Least Squares) IK — full online IK when G1 MJCF is loaded

Keyframe fallback — 3 pre-baked poses interpolated linearly (home / reach_table / drop_bin)

Joint limits enforced for all 7 right-arm joints per G1 spec

POC simplification: pre-computed waypoint poses are sufficient; exact IK precision is not critical

## Step 7 — Inspection Control Loop  (2 h)

robot/inspection_controller.py implements a 5-state machine:

WATCHING     → run YOLOv8 on head cam every 200 ms

↓ defect detected (confidence > 0.45)

APPROACHING  → blend arm to reach_table pose  (1.5 s)

PICKING      → hold reach pose, simulate grasp  (1.0 s)

DROPPING     → blend arm to drop_bin pose  (1.0 s)

RETURNING    → blend arm back to home  (1.2 s)

↓

WATCHING     (loop)

All transitions are timed (no joint-space feedback required for POC)

State machine decoupled from viewer loop — call step(dt, frame) each sim step

## Step 8 — Main Demo  (2 h)

demo.py launches MuJoCo interactive viewer (3D) + OpenCV head-cam window side by side

Parts spawn on table every 5 seconds (cycling through all 3 classes)

Detection bounding boxes drawn on OpenCV window with class label and confidence

Console prints state transitions and 30-second metric summaries

# Standalone scene (no G1 MJCF needed)

python demo.py --scene standalone

# Full scene with G1

python demo.py --scene full

# Headless (saves annotated frames to output\)

python demo.py --scene standalone --no-viewer

## Step 9 — Demo Polish  (1 h)

Record demo video using OBS Studio or Windows Game Bar (Win+G)

Prepare 5-slide deck: Problem → Solution → Architecture → Live Demo → Next Steps

Highlight Cosmos 3 augmentation slide: sim-to-real gap bridged via cloud API

# 8. Success Criteria

| Metric | Target | Measurement |
| --- | --- | --- |
| Defect detection mAP | > 80% | YOLOv8 validation set at end of training |
| False positive rate | < 20% | Good parts incorrectly flagged as defects |
| Arm reach success | > 70% of defects | Arm visibly reaches table area on detection |
| Inference latency | < 150 ms/frame | Detector.detect() wall-clock time logged |
| Full loop stability | 5 min crash-free | demo.py running in viewer without exception |

# 9. Key Commands

## Day 1 — Environment & Training

cd C:\huminoid

python -m venv .venv

.venv\Scripts\activate

pip install -r requirements.txt

# Verify install

python verify_setup.py

# Generate training data

python vision\generate_data.py --n 2000

# Optional: Cosmos augmentation

$env:NVIDIA_API_KEY = "nvapi-xxxx"

python vision\augment_cosmos.py --n 500

# Train YOLOv8n

python vision\train_yolo.py --epochs 50 --batch 8

## Day 2 — Demo

# Quick scene test (no G1 MJCF required)

python -m mujoco.viewer --mjcf scene\inspection_scene_standalone.xml

# Full demo

python demo.py --scene standalone

# With G1 MJCF (after cloning unitree_mujoco)

python demo.py --scene full

# 10. Risks & Mitigations

| Risk | Likelihood | Mitigation |
| --- | --- | --- |
| G1 MJCF model fails to load | Medium | Use inspection_scene_standalone.xml — placeholder robot demonstrates full pipeline |
| YOLOv8 CPU inference too slow (>200 ms) | Low | Drop to 5 fps inference; load ONNX model instead of .pt; demo still looks smooth |
| IK does not converge on table targets | Medium | Pre-baked keyframe poses (home/reach_table/drop_bin) are used by default — exact IK not required |
| 16 GB RAM pressure during training | Low | Reduce batch to 4; reduce dataset to 1,000 images; disable cache=True in train_yolo.py |
| Windows path issues with MuJoCo | Low | Project at C:\huminoid (short path). E:\ drive is full — do not use E:\ paths |

# 11. Future Upgrade Path

When NVIDIA GPU access is available (cloud VM or local hardware):

| Phase | Action | Benefit |
| --- | --- | --- |
| Post-POC | Port scene to NVIDIA Isaac Sim | RTX ray tracing, photorealistic rendering, ROS2 integration |
| Post-POC | Run Cosmos 3 Nano (16B) locally | Faster augmentation, private, no API cost or rate limits |
| Month 1–2 | Real robot deployment on physical G1 | Transfer trained YOLOv8 weights + IK controller to hardware |
| Month 2–3 | Conveyor belt scene + online retraining | Continuous-flow inspection; model improves from real detections |