Product Requirements Document

Humanoid Robot Industrial Inspection POC

| Product | Humanoid Robot Inspection POC |
| --- | --- |
| Version | 1.0 |
| Status | In Development |
| Owner | Mor Yegerman |
| Date | June 2026 |
| Horizon | 2-day POC; demo ready for stakeholder presentation |

# 1. Executive Summary

This POC demonstrates that a humanoid robot (Unitree G1) can perform automated visual quality inspection in a simulated industrial environment, providing a compelling proof of concept for deploying humanoid robots in manufacturing inspection cells. The system detects defective parts via a head-mounted camera, classifies them using a fine-tuned computer vision model, and physically removes them with the robot arm — closing a full autonomous inspection loop entirely in simulation, on commodity Windows PC hardware.

The primary goal is a 5-minute stable demo ready for a stakeholder presentation. The secondary goal is to establish the software architecture and training pipeline that can be transferred to a real G1 robot post-POC.

# 2. Problem Statement

## 2.1  Business Problem

Manual quality inspection on production lines is labour-intensive, inconsistent, and difficult to scale. Humanoid robots offer the dexterity to operate in existing work cells without retooling, but deploying them requires demonstrating a credible software stack before committing to hardware investment.

## 2.2  Technical Problem

Existing humanoid robot demo stacks (Isaac Sim + ROS2) require NVIDIA GPUs and significant infrastructure. This POC must run on commodity hardware (AMD GPU, Windows, no ROS2) within a 2-day build timeline, while still producing a convincing and technically sound demo.

# 3. Goals and Non-Goals

## 3.1  Goals

Demonstrate a G1 robot detecting and removing defective parts in simulation

Achieve >80% defect detection mAP on held-out synthetic validation set

Run the full demo loop stably for at least 5 minutes without crashing

Show a plausible path to real robot deployment (same code, different scene)

Produce a shareable demo video and stakeholder presentation deck

## 3.2  Non-Goals (POC scope)

Real robot hardware integration — simulation only for this POC

Conveyor belt / moving parts — static table with periodic spawning is sufficient

ROS2 integration — direct MuJoCo Python bindings only

Real-world generalisation — training data is 100% synthetic

Multi-robot / multi-arm scenarios

Production-grade error recovery or safety systems

# 4. User Stories

| ID | As a... | I want to... | So that... | Priority |
| --- | --- | --- | --- | --- |
| US-01 | Demo presenter | See the robot arm move to pick a defective part | Stakeholders understand the autonomous loop | P0 |
| US-02 | Demo presenter | See bounding boxes on the camera feed in real time | The vision system is visually convincing | P0 |
| US-03 | Demo presenter | See good parts ignored and only defects picked | The classification is clearly working | P0 |
| US-04 | Demo presenter | Run the demo for 5+ minutes without a crash | The system appears production-ready | P0 |
| US-05 | Demo presenter | Show a metrics printout (inspected/defects/picks) | ROI story is quantified | P1 |
| US-06 | Engineer | Run the full pipeline from scratch in one day | Reproduce and extend the work | P1 |
| US-07 | Engineer | Swap in the real G1 MJCF without rewriting control code | Path to real hardware is clear | P1 |
| US-08 | Stakeholder | See a recorded demo video | Evaluate without attending a live session | P2 |

# 5. Functional Requirements

## 5.1  Vision

| ID | Requirement | Priority | Acceptance Criteria |
| --- | --- | --- | --- |
| FR-V1 | Classify parts into 3 classes: good, defect_crack, defect_discolor | P0 | mAP > 80% on val set |
| FR-V2 | Run inference on head-cam frames at >= 5 fps | P0 | Latency < 200 ms per frame on i7-6700 |
| FR-V3 | Draw bounding boxes and confidence on OpenCV window | P0 | Boxes visible on every detection |
| FR-V4 | Only trigger arm action on confidence >= 0.45 | P0 | False positive rate < 20% |
| FR-V5 | Load ONNX model when available (faster than .pt on CPU) | P1 | ONNX loaded by default if best.onnx exists |

## 5.2  Control

| ID | Requirement | Priority | Acceptance Criteria |
| --- | --- | --- | --- |
| FR-C1 | State machine transitions: WATCHING→APPROACHING→PICKING→DROPPING→RETURNING | P0 | All 5 states reachable in single run |
| FR-C2 | Arm reaches work table area on APPROACHING | P0 | Gripper visually moves toward table (>70% attempts) |
| FR-C3 | Part removed from table during PICKING state | P0 | Part body moves off-scene after pick |
| FR-C4 | Arm returns to home pose after each pick cycle | P0 | Home pose reached before next WATCHING |
| FR-C5 | Controller works with standalone scene (no G1 MJCF) | P1 | demo.py --scene standalone runs without error |

## 5.3  Simulation

| ID | Requirement | Priority | Acceptance Criteria |
| --- | --- | --- | --- |
| FR-S1 | New part spawns on table every 5 seconds | P0 | Part body position updated; visible in viewer |
| FR-S2 | 3 part classes visually distinguishable | P0 | Green / red / dark grey colours in scene |
| FR-S3 | Reject bin positioned within arm reach | P0 | Arm drop pose places part over bin |
| FR-S4 | Head camera field of view covers table surface | P0 | Table visible in head-cam render at all times |

## 5.4  Training Pipeline

| ID | Requirement | Priority | Acceptance Criteria |
| --- | --- | --- | --- |
| FR-T1 | Generate >= 2,000 synthetic labeled frames | P0 | data/images/train + val populated after generate_data.py |
| FR-T2 | Auto-label from camera projection (no manual annotation) | P0 | All images have paired .txt label file |
| FR-T3 | YOLOv8n trains to >80% mAP in <= 2 hours on CPU | P0 | metrics/mAP50 printed at end of training |
| FR-T4 | Export trained model to ONNX | P1 | best.onnx present after train_yolo.py |
| FR-T5 | Cosmos augmentation integrates into training set | P2 | Augmented images merged before train when present |

# 6. Non-Functional Requirements

| Category | Requirement | Target |
| --- | --- | --- |
| Performance | Detection latency | < 150 ms / frame on i7-6700 (CPU) |
| Performance | Physics simulation rate | >= 250 Hz (MuJoCo default timestep=0.004) |
| Stability | Demo run duration without crash | 5 minutes minimum |
| Compatibility | GPU | AMD RX 580 — no CUDA required at runtime |
| Compatibility | OS | Windows 10 64-bit |
| Reproducibility | Full pipeline re-run time | < 3 hours (generate + train + demo) |
| Portability | Scene upgrade path | Full G1 MJCF scene loads without code changes |
| Storage | Total project disk usage | < 3 GB including trained model and dataset |

# 7. Milestones & Timeline

| Milestone | Description | Target | Status |
| --- | --- | --- | --- |
| M1 — Environment | Python venv + deps installed; verify_setup.py passes | Day 1, Hour 1 | Done |
| M2 — Scene | Standalone scene loads in MuJoCo viewer | Day 1, Hour 2 | Done |
| M3 — Data | 2,000 labeled frames generated | Day 1, Hour 4 | Done |
| M4 — Training | YOLOv8n reaches >80% mAP; best.onnx exported | Day 1, End | In Progress |
| M5 — Control loop | demo.py runs; arm moves on detection | Day 2, Hour 3 | Pending |
| M6 — Demo polish | 5-min stable run; video recorded; deck ready | Day 2, End | Pending |
| M7 — Full G1 scene | G1 MJCF loaded; demo.py --scene full runs | Post-Day 2 | Pending |

# 8. Risks

| Risk | Impact | Likelihood | Mitigation |
| --- | --- | --- | --- |
| G1 MJCF doesn't load on Windows | High | Medium | Standalone scene is complete fallback — full demo possible without G1 |
| YOLOv8 mAP < 80% after 50 epochs | Medium | Low | Increase epochs to 100; add colour jitter augmentation in generate_data.py |
| CPU inference too slow for live demo | Medium | Low | ONNX model + 5fps cap; demo still looks smooth at 10fps display |
| 16 GB RAM exhausted during training | High | Low | batch=4; cache=False; reduce dataset to 1,000 images |
| Arm doesn't reach table visually | High | Medium | Tune reach_table keyframe pose angles in ik_solver.py |
| Cosmos API key unavailable | Low | Medium | Skip augmentation — 2,000 synthetic frames alone achieve >80% mAP target |

# 9. Definition of Done

The POC is considered complete when all P0 acceptance criteria are met:

verify_setup.py reports all checks passed

generate_data.py produces 2,000 images with paired labels

train_yolo.py reports mAP50 >= 0.80 on validation set

demo.py --scene standalone runs for 5 minutes without exception

Arm visibly moves to table on >= 70% of defect detections

OpenCV window shows bounding boxes with correct class labels

Demo video recorded (OBS / Windows Game Bar)

# 10. Post-POC Roadmap

| Phase | Feature | Dependency |
| --- | --- | --- |
| v1.1 | Real G1 MJCF scene (23-DOF) | Clone unitree_mujoco; adjust arm keyframes |
| v1.2 | Online Cosmos augmentation loop | NVIDIA_API_KEY; augment_cosmos.py already written |
| v2.0 | Isaac Sim port | NVIDIA GPU (cloud VM or local RTX) |
| v2.0 | ROS2 integration | Ubuntu + ROS2 Humble + MoveIt |
| v2.1 | Conveyor belt scene | Custom MuJoCo conveyor geom + velocity actuator |
| v3.0 | Real G1 hardware deployment | Physical Unitree G1 robot; transfer ONNX weights |